Standard Preserved Moss Wall

Section 09 77 53

3-Part MasterFormat Document

Part 1 General

1.1 SECTION INCLUDES

- Custom Moss Walls

1.2 RELATED SECTIONS

- Section 06 10 00 “Rough Carpentry” for coordination of host wall preparation.

- Section 09 29 00 “Gypsum Board” for coordination of host wall preparation.

- Section 06 20 00 “Finish Carpentry" if a third-party carpenter is providing a custom frame around the moss.

- Section 26 05 00 “Common Work Results for Electrical” for the power hookups needed for backlit signage (only if backlit signage is specified).

1.3 REFERENCES

- Build America, Buy America (BABA) Act: All manufactured products incorporated into this project shall be produced in the United States.

  - Manufactured products shall be produced in the U.S. and the cost of the components of the manufactured product that are mined, produced, or manufactured in the U.S. shall be greater than 55 percent of the total cost of all components.

  - Contractor shall provide Manufacturer’s Certification of BABA Compliance for all products provided under this Section.

1.4 SUBMITTALS

- Submit in accordance with administrative requirements.

- Product Data:

  - Safety Data Sheets.

  - Preparation instructions and recommendations.

  - Installation methods and recommendations.

  - Storage and handling requirements.

  - Product certifications.

- Verification Samples

- Shop Drawings:

  - Moss wall design is finalized in a designated call with a GrowUp Designer. Shop drawings are produced using final as-built dimensions.

1.5 QUALITY ASSURANCE

- Manufacturers Qualifications: GrowUp Greenwalls is a company specializing in manufacturing products specified in this section with a minimum five years documented experience.

- Installer Qualifications: GrowUp Greenwalls is a company specializing in performing Work of this section with minimum five years documented experience with projects of similar scope and complexity.

- Single Source: All panels and accessories from a single source for uniformity. :

1.6 PRE-INSTALLATION CONFERENCE

- Conduct a meeting with Architects, Contractor, and related trades approximately two weeks prior to scheduled Work. Agenda shall include schedule, responsibilities, coordination, and installation requirements.

1.7 DELIVERY, STORAGE, AND HANDLING

- Inspect delivered materials for damage in shipping.

- Store in protected conditions per manufacturer’s recommendations.

  - Protect against damage due to weather, dampness, excessive temperature, and construction operations.

- Do not install damaged or defective materials.

1.8 PROJECT CONDITIONS

- Maintain conditions (temperature, humidity, and ventilation) within recommended limits set by the manufacturer.

- Host wall must be in the condition specified in the submittal, whether the Moss Wall will be attached to wood blocking or drywall.

- Protect installed material from dust and damage from surrounding construction.

1.9 WARRANTY

- Manufacturer’s standard limited warranty unless otherwise indicated.

Part 2 Products

2.1 MANUFACTURERS

- Acceptable Manufacturer: GrowUp Greenwalls LLC.

  - Location: 1283 Simpson Way, Escondido, CA 92029

  - Phone: 1.800.833.4890

  - Email: info@growup.green

  - Web Address: www.growup.green

- Substitution: Not permitted.

2.2 MATERIALS

- Design Requirements:

  - Preserved Moss Varieties

  - Included in the Work: Following the approved drawings.

    - Moss wall preparation.

    - Sign components; optional.

    - Approved Frames; optional.

    - Lighting; optional.

  - Moss Wall System: Designed to be permanently installed.

    - Moss is applied to plywood, fire-rated plywood, or fire-rated acoustical felt substrate with adhesive.

    - Basis of Design: Moss Walls manufactured by GrowUp Greenwalls LLC. Harvested moss undergoes a preservation process using Glycerine (CAS 56-81-5) and water-based dyes.

      - Weight: 1.5-3 lbs per square foot. Precise weight depends on moss variety and backing substrate.

      - Species of Moss:

        - Mood Moss

        - Reindeer Moss

        - Deerfoot Moss

        - Carpet Moss

        - Pin Cushion Moss

        - Spanish Moss

      - Additional Design Elements:

        - Preserved plants

        - Artificial plants

        - Other

      - Frame Type:

        - Wood Frame

          - Natural

          - Matte white

          - Matte black

        - Aluminum

          - Matte white

          - Matte black

          - Brushed silver

        - Custom frame

      - Signage:

        - Acrylic laser cut and mounted on stand-offs

        - Printed on clear acrylic

        - Acrylic with backlighting

        - Custom signage

2.3 ACCESSORIES

- Mounting hardware, brackets, and fasteners compatible with the host wall.

2.4 FABRICATION

- Panels are sized to optimize shipping costs and installation requirements.

- Panel edges shall be finished on-site to achieve a seamless look.

Part 3 Execution

3.1 EXAMINATION

- Verify host wall substrate conditions suitable for installation.

3.2 PREPARATION

- Clean and prepare surfaces per manufacturer’s instructions.

3.3 INSTALLATION

- Installation of the Moss Wall should be scheduled after painting is complete. Dusty conditions are harmful to the panels and can create additional costs in cleaning of the Moss Wall.

- Beginning of installation constitutes acceptance of substrate conditions.

- Install panels per approved drawings and manufacturer’s recommendations.

- Secure panels at minimum four attachment points per panel.

  - Fasteners are selected based on the host wall substrate provided by the General Contractor.

- Adjust material to ensure uniform coverage and blended appearance as per manufacturer’s standards.

3.4 FIELD QUALITY CONTROL

- Inspect completed installation for integrity, visual consistency, and accuracy to intended design.

3.5 CLEANING AND PROTECTION

- Remove installation debris.

3.6 CLOSEOUT

- Provide maintenance instructions and warranty documents.

- Conduct final inspection with the Owner, Architect, or General Contractor.

END OF SECTION